Table of Contents

# NIST 800-53 Rev. 5 Control Mappings

This document provides a high-fidelity mapping of the uiao-core architecture planes to the NIST 800-53 Rev. 5 control families. These mappings are critical for the agency’s FedRAMP 20x Phase 2 Pilot (concluding March 31, 2026) and establish the compliance baseline for the transition to Phase 3 in Q3 2026.

## 1. Identity Plane (Zero-Trust Core)

Focus: Access Control (AC) and Identification and Authentication (IA)

| Control ID | Control Name | uiao-core Implementation |
| --- | --- | --- |
| AC-2 | Account Management | Centralized Azure AD integration for all network administrative and user accounts. |
| AC-3 | Access Enforcement | ISE-driven SGT assignment ensures privilege-based access at the network ingress. |
| IA-2 | Identification and Authentication (Org Users) | Enforces MFA and certificate-based authentication (EAP-TLS) via the Identity Plane. |
| IA-8 | Identification and Authentication (Non-Org Users) | Automated Guest-access workflows via ISE, providing strict isolation for contractor/external entities. |

## 2. Addressing Plane (Visibility and Inventory)

Focus: Configuration Management (CM) and System and Comm. Protection (SC)

| Control ID | Control Name | uiao-core Implementation |
| --- | --- | --- |
| CM-2 | Baseline Configuration | appendices.yml acts as the immutable baseline for all site IP and DNS configurations. |
| CM-8 | System Component Inventory | Automated Infoblox IPAM provides a real-time, authoritative inventory of every connected endpoint. |
| SC-20 | Secure Name / Address Resolution | DNSSEC and automated DNS zone protection (Appendix A-04) ensure the integrity of the name resolution plane. |
| SC-21 | Secure Name / Address Resolution (Recursive) | Centralized DNS hierarchy prevents DNS poisoning and ensures recursive queries follow agency-approved paths. |

## 3. Overlay Plane (Segmentation and Isolation)

Focus: System and Communications Protection (SC)

| Control ID | Control Name | uiao-core Implementation |
| --- | --- | --- |
| SC-7 | Boundary Protection | Overlay fabric utilizes VRFs (Macro-segmentation) to create immutable boundaries between security zones. |
| SC-39 | Process Isolation / Segmenting | SGACLs (Appendix BA-02) enforce micro-segmentation, preventing lateral movement between disparate host roles. |
| SC-8 | Transmission Confidentiality and Integrity | SD-WAN Edge (Appendix AA-02) mandates AES-256-GCM encryption for all inter-site data in transit. |

## 4. Telemetry Plane (Accountability and Monitoring)

Focus: Audit and Accountability (AU) and System and Info Integrity (SI)

| Control ID | Control Name | uiao-core Implementation |
| --- | --- | --- |
| AU-2 | Event Logging | Automated Splunk integration captures every CLI change and automated API call in the uiao-core ecosystem. |
| AU-6 | Audit Record Review, Analysis, and Reporting | Closed-loop feedback system correlates identity (SGT) with traffic logs for rapid incident reconstruction. |
| SI-4 | System Monitoring | Real-time telemetry via Azure Monitor provides continuous monitoring of infrastructure health and security posture. |

## FedRAMP 20x Phase 2 Compliance Note

As of March 21, 2026, these mappings have been validated against the current Pilot requirements. Full operationalization of these controls is a prerequisite for the Q3 2026 transition to FedRAMP 20x Phase 3.